ALLEN 8005571.05 Oil Level sensor

| REVISION HISTORY |  |  |
| --- | --- | --- |
| REV | ECO | DATE |
| A | ADCN 01894 | 10/22/2021 |
| B | ADCN 01993 | 3/9/2022 |
| C | ADCN 02091 | 8/4/2022 |
| D | ADCN 02339 | 06/21/2023 |
| E | ADCN 02429 | 09/11/2023 |

Introduction:

The following procedure sets forth the method for conducting acceptance tests on production units of the Allen 8005571.05 (P&WC 30Y0241-01) Oil Level Sensor. Each Oil Level Sensor shall be subjected to and meet the requirements of this document before being accepted for use.

Reference Documents:

NOTE:	Latest Revision to be used unless otherwise specified

Equipment Specification ES 3582 Sensor – Oil Level

General Specification ES 2968

Acronyms:

SO 		Shipping Order

SA INSP 	Sub-Assembly Inspection

FA INSP 	Final-Assembly Inspection

Equipment Required:

NOTE:	Equivalent substitutes may be used provided the equipment has the same or better accuracies.

0-8.8 Lb Scale, Accuracy > ± .005 Lbs.

Oil Level Test Sump TU-002-V3

Oil Level Test Bench T-20037

Misc. Shut Off Valves and Plumbing Supplies

Hypot III Dielectric Withstand Tester, Associated Research, Inc., Model #3770, Accuracy within 2% of Setting, Allen ID No. 9631887

Micro-ohmmeter, AEMC Instruments Model 6240, Allen ID No. 112537SBDV

Analog Laser Sensor, Keyence, IL-600, LK-503, or LK-G507 (Linearity ±0.25% of Full Scale)

Test Fixture T-20079

0-500 PSI Pressure Gage, Accuracy: ±0.1% of Reading

Test Conditions:

Cleaning:

Prior to testing the unit, all parts normally in contact with the fluid use in service shall be thoroughly cleaned to remove all foreign matter.

Test Fluid(s):

MIL-PRF-7808 or MIL-PRF-23699

Temperature:

Ambient and Test Fluid: 60° to 100° F.

Test Facilities:

Testing required in this Acceptance Test Procedure shall be conducted at the following location:

Allen Aircraft Products, Inc.

6168 Woodbine Avenue

Ravenna, OH  44266

Phone: (330) 296-9621

367833.70 Switch and Resistor Assembly Procedure:

Record the resistance between the Channel A Orange wire and Black wire. Resistance shall be 2100± 21 ohms. Record the resistance between the Channel B Red wire and Yellow wire. Resistance shall be 2100± 21 ohms.

8005571.05 Oil Level Sensor Procedure:

Examination of Product: Sample 100%

Examine each unit to determine conformance with Allen Drawing 8005571.05 with respect to design, dimensions, materials, finishes, workmanship, and any other requirements for which there are no specific functional tests. Examine the part marking to determine the conformance with Allen Process Drawing 402562.70. The Oil Level Sensor float shall move back and forth freely with no binding or sticking. Record "OK" on Data Card if unit is acceptable.

Weight: Sample 100%

Weigh dry, completed unit including markings, coatings, etc. in accordance with above sampling plan. Unit weight shall not exceed .82 lb. Record actual weight on Data Card.

Insulation Resistance: Sample 100%

Connect pins 2, 3, 4, & 5 to the positive supply from the Dielectric Withstand Tester. Connect the ground to the flange of the Oil Level Sensor. See Figure 1 and Figure 2 for test setup. Raise the voltage to 475-525 VDC and record the resistance reading. The minimum resistance shall be 40 megaohms. Lower the voltage to zero. Record resistance on Data Card.

Connect pins 2 & 3 to the positive supply from the Dielectric Withstand Tester and ground pins 4 & 5 as shown in Figure 7 and Figure 8. Raise the voltage to 475-525 VDC and record the resistance reading. The minimum resistance shall be 40 megohms. Lower the voltage to zero. Record resistance on Data Card.

Dielectric Strength: Sample 100%

*If any reruns of the dielectric strength test are required, these tests shall be run at 80% voltage (859 volts RMS).

Pins To Flange

Connect pins 2, 3, 4, & 5 to the AC Outlet of the Dielectric Withstand Tester and ground the flange of the Oil Level Sensor, as shown in Figure 1 and Figure 2. Raise the voltage at a uniform rate of 250 to 500 volts per second to 1073* volts RMS minimum at 60 Hz. Maintain the voltage at 1073* volts RMS minimum for a minimum of one second and then lower the voltage at a uniform rate of 250 to 500 volts per second. Any arcing as evidenced by flashover, sparkover, breakdown or leakage current exceeding 2 milliamperes, shall constitute failure of the test.

Channel To Channel

Connect pins 2 & 3 to the AC Outlet of the Dielectric Withstand Tester and ground pins 4 & 5 as shown in Figure 7 and Figure 8. Raise the voltage at a uniform rate of 250 to 500 volts per second to 1073* volts RMS minimum at 60 Hz. Maintain the voltage at 1073* volts RMS minimum for a minimum of one second and then lower the voltage at a uniform rate of 250 to 500 volts per second. Any arcing as evidenced by flashover, sparkover, breakdown or leakage current exceeding 2 milliamperes, shall constitute failure of the test. Record "OK" on Data Card if unit is acceptable.

Bonding Resistance: Sample 100%

Connect one lead of the Digital Multimeter to the connector shell. Connect the other lead to the bottom side of the mounting flange (Datum A on drawing 8005571.05). See Figure 3 and Figure 4 for test setup. Measure and record the bonding resistance on the data card. The bonding resistance between the connector shell and mounting flange shall not exceed 10 milliohms.

Fluid Level: Sample 100%

Install Oil Level Sensor into the Oil Level Test Sump TU-002-V3 or T-20037 as shown in Figure 5. Start with oil level below the bottom of the test unit. Connect -10, 6 pin, mating plug to Oil Level Sensor Connector and connect proper lead wires to the Micro-Ohmmeter. Fill the sump so that the resistance reading between Signal (Pin 2) and Return (Pin 3) and between Signal (Pin 5) and Return (Pin 4) is 1100 ± 11 ohms. Record resistance on Data Card. Then gradually drain the sump so that resistance reads 100 ± 2 ohms.

Begin the oil level test with the oil level below the bottom of the oil level sensor. Begin testing Circuit A Signal (Pin 2) and Return (Pin 3). Add oil to the sump to increase oil level. During the oil rising test, stop the pump at each oil level required in Table 1 and measure electrical resistance between the applicable pins. Record in Table 1 if the electrical resistance is acceptable. After verifying all oil level rising requirements, continue adding oil until the fluid level measures 2.750 inches maximum. Then gradually drain the sump to decrease the oil level. During the oil draining test, stop the pump at each oil level required in Table 1 and measure electrical resistance between the applicable pins. Record in Table 1 if the electrical resistance is acceptable.

With the oil level below the bottom of the oil level sensor. Begin testing Circuit B Signal (Pin 5) and Return (Pin 4). Add oil to the sump to increase oil level. During the oil rising test, stop the pump at each oil level required in Table 2 and measure electrical resistance between the applicable pins. Record in Table 2 if the electrical resistance is acceptable. After verifying all oil level rising requirements, continue adding oil until the fluid level measures 2.750 inches maximum. Then gradually drain the sump to decrease the oil level. During the oil draining test, stop the pump at each oil level required in Table 2 and measure electrical resistance between the applicable pins. Record in Table 2 if the electrical resistance is acceptable.

Note:	The Oil Level Sensor is designed to change electrical resistance readings within the ranges in Table 1 and Table 2. For example, for circuit A, as oil is rising, the Oil Level Sensor should read 200 ± 3 ohms between 7.460 and 7.560 inches. The technician may verify the 200 ± 3 ohms reading at any point between 7.460 and 7.560 inches.

Record “OK” for accept or “FAIL” for reject on the Data Sheet based on meeting the Table 1 and Table 2 requirements. If an electrical resistance is observed which is outside the electrical resistance tolerance levels permitted in Table 1 or Table 2, stop the test stand pump. With the oil level constant, if the observed electrical resistance tolerance is outside the resistance levels permitted in Table 1 or Table 2, this shall be cause for rejection of the Oil Level Sensor. With the oil level constant, if the electrical resistance tolerance meets Table 1 and Table 2 requirements, continue testing.

If the test operator inadvertently overshoots any oil level set point, the operator shall set the oil level to the last successfully met oil level. The operator shall then continue with the test.

Note: Oil fill and drain rates shall not exceed 1000 cc/min (33.81 oz/min). Fluid level ATP testing is performed at room temperature.

Proof Pressure: Sample 100%

Install Oil Level Sensor into Fixture T-20079 as shown in Figure 6. Apply 103 PSI with hand pump. Close shut off valve. After 1 minute there shall be no decrease in pressure. Open the shut off valve, reduce pressure, and remove Oil Level Sensor from test fixture. Record “0” on data sheet if unit is acceptable.

Note: The Oil Level Sensor has a proof pressure of 80 PSI and a short time maximum temperature of 335°F. Because this test will be performed at ambient lab conditions, the test pressure has been increased to compensate for the reduced material yield strength at 335°F. According to MMPDS-10, the yield strength of 6061-T651 Aluminum is 78% of its ambient temperature yield strength (see Figure 9). To calculate the compensated proof pressure requirement, the following equation is used.

Proof PressureComp= Proof PressureAmb% Yield StrengthOp. Temp=80 PSI78%=102.56 PSI

| Switch Location | Oil Rising |  |  | Oil Draining |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Oil Level from Mounting Surface Set Point (inches) | Resistance Requirement (Ohms) | OK? Y/N | Oil Level from Mounting Surface Set Point (inches) | Resistance Requirement (Ohms) | OK? Y/N |
| 1A | 7.910-8.010 | 100 ±2 |  | 7.550-7.650 | 100 ±2 |  |
| 2A | 7.460-7.560 | 200 ±3 |  | 7.100-7.200 | 200 ±3 |  |
| 3A | 7.010-7.110 | 300 ±4 |  | 6.650-6.750 | 300 ±4 |  |
| 4A | 6.560-6.660 | 400 ±5 |  | 6.200-6.300 | 400 ±5 |  |
| 5A | 6.110-6.210 | 500 ±6 |  | 5.750-5.850 | 500 ±6 |  |
| 6A | 5.660-5.760 | 600 ±7 |  | 5.300-5.400 | 600 ±7 |  |
| 7A | 5.210-5.310 | 700 ±8 |  | 4.850-4.950 | 700 ±8 |  |
| 8A | 4.760-4.860 | 800 ±9 |  | 4.400-4.500 | 800 ±9 |  |
| 9A | 4.310-4.410 | 900 ±10 |  | 3.950-4.050 | 900 ±10 |  |
| 10A | 3.860-3.960 | 1000 ±11 |  | 3.500-3.600 | 1000 ±11 |  |
| 11A+12A | 3.410-3.510 | 1100 ±12 |  | 3.050-3.150 | 1100 ±12 |  |

TABLE 1

Oil Switching Levels - Circuit A Switches

| Switch Location | Oil Rising |  |  | Oil Draining |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Oil Level from Mounting Surface Set Point (inches) | Resistance Requirement (Ohms) | OK? Y/N | Oil Level from Mounting Surface Set Point (inches) | Resistance Requirement (Ohms) | OK? Y/N |
| 1B+2B | 7.685-7.785 | 100 ±2 |  | 7.325-7.425 | 100 ±2 |  |
| 3B | 7.235-7.335 | 200 ±3 |  | 6.875-6.975 | 200 ±3 |  |
| 4B | 6.785-6.885 | 300 ±4 |  | 6.425-6.525 | 300 ±4 |  |
| 5B | 6.335-6.435 | 400 ±5 |  | 5.975-6.075 | 400 ±5 |  |
| 6B | 5.885-5.985 | 500 ±6 |  | 5.525-5.625 | 500 ±6 |  |
| 7B | 5.435-5.535 | 600 ±7 |  | 5.075-5.175 | 600 ±7 |  |
| 8B | 4.985-5.085 | 700 ±8 |  | 4.625-4.725 | 700 ±8 |  |
| 9B | 4.535-4.635 | 800 ±9 |  | 4.175-4.275 | 800 ±9 |  |
| 10B | 4.085-4.185 | 900 ±10 |  | 3.725-3.825 | 900 ±10 |  |
| 11B | 3.635-3.735 | 1000 ±11 |  | 3.275-3.375 | 1000 ±11 |  |
| 12B | 3.185-3.285 | 1100 ±12 |  | 2.825-2.925 | 1100 ±12 |  |

TABLE 2

Oil Switching Levels - Circuit B Switches

FIGURE 1

Dielectric Strength and Insulation Resistance Test Setup

Return Output Terminal

High Voltage Output Terminal

Alligator clamp to mounting surface, Datum A on 8005571.05

-10, 6 pin, mating plug or equivalent

FIGURE 2

Dielectric Strength and Insulation Resistance Setup Schematic

FIGURE 3

Bonding Resistance Test Setup

Alligator clamp to connector shell

Alligator clamp to mounting surface, Datum A on 8005571.05

FIGURE 4

Bonding Resistance Setup Schematic

FIGURE 5

Oil Level Functional Test Setup

FIGURE 6

Proof Pressure Test Setup

FIGURE 7

Dielectric Strength and Insulation Resistance Test Setup

Return Output Terminal

Alligator clamp to pins 2 & 3.

High Voltage Output Terminal

Alligator clamp to pins 4 & 5.

-10, 6 pin, mating plug

FIGURE 8

Dielectric Strength and Insulation Resistance Setup Schematic

FIGURE 9

Yield Strength Reduction (Figure 3.6.2.2.1(b) from MMPDS-10)